Amtrak ID 800188

Navy Inspection Car 118

The 118 is currently store at Fort Eustis Virginia. The currents owners Mr. and Mrs. Tyler B. Robbins. 1631 Epping Road Gates Mills, Ohio 44040.

Car history.

- Builder Plans 2843 Lot 4291 			              1914-Dec 1915

- Built for Union Pacific Chair Car      	 	UP 741		Dec 1915 -April 1916

- Renumbered UP Chair Car   			UP 1210  	Apr 1916 -1938

- Remodeled to a UP Business Car 		UP 118		Jan 1938 - Dec 1951

- Renumbered UP Business car		UP 114 (2nd)	Dec 1951 - Apr 1957

- Renumbered UP Business car 		UP 120 (3rd)	Apr 1957 - Jun 1970

- Retired June 1970 						Jun 1970

- Purchase by Tyler B. Robbins from Morriston & Erie RR 1978

- Moved to Cleveland Ohio Collinwood Railroad yard

Restoration 1983-1988 Cleveland, Ohio Collinwood Rail Yard.

- Amtrak 40 year rebuild performed on original 3 axle bolt on pedestal trucks.

- Lighting electrical system updated to 120 volt AC from 32 volt DC battery supplied power.

- Amtrak Head End Power (HEP) and 27 point communications installed.

- Original UC brake system overhauled per Amtrak/AAR and FRA standards.

- Perkins generator added to car.

- Ice operated air conditioning upgraded to R-12 refrigerant system.

- Heating system upgraded to oil fired boiler, and electric baseboard heating.

- Complete interior restoration.

- Water raising system replaced and modernized.

- Microphor toilet system with holding tanks added to meet Amtrak requirements.

- All exterior windows replaced with new window frames and laminated safety glass.

- Car repainted to present color.

Upgrade and restoration completed April 2005. Work performed Toledo, Ohio.

- Car converted from 3 axle bolt on pedestal trucks to 2 axle cast pedestal thread brake trucks.

- Complete remanufacture of two axle trucks per Amtrak specs.

- Timken 6-1/2 X 12 EE bearings

- Center bowl attachment replaced with central bearing to mate new trucks to car body.

- UC brake system replaced with 26CV8 brake system, all airbrake piping on the car removed and replaced with welded sch 80 piping per AAR requirements.

- Removal and replacement of handbrake to new AAR/FRA standards.

- Replacement of control slack coupler and draft gear. Replaced with “Tightlock” HT81 couplers, draft gears.

- New spring-loaded coupler carriers.

- Air conditioning system replaced with modern materials replacing obsolete R-12 refrigerant.

- Head End Power (HEP) receptacles and pigtails replaced.

PC-1 to be performed George Payne late February 2022 location Fort Eustis

PC-2 Date 4/21/05 Toledo, Ohio

PC-2A Date 9/27/13 Amtrak Beech Grove