# WHEEL & TRUCK REPORT				LOCOMOTIVE:  ACWR 928

# EMPLOYEE:       R.Dickson

# ACWR  Form 7070						DATE: 	   8/24/2022

|  | Wheel Thickness (229.75(j)) |  | Flange Height (229.75(h)) |  | Flange Thickness (229.75(f)) |  |  | Witness Reading (229.73) |  | Shims Installed (229.73(b)) |  |  | Pedestal Jaw Wear |  |  |  |  |  | Side Bearing Clearance (229.69(b)) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L#1 | 39 |  | 21 |  | 24 |  |  | 28 |  | 0 |  |  | 7 |  |  |  |  |  | RF |  |  |  |  | 2 |
| L#2 | 41 |  | 18 |  | 22 |  |  | 31 |  | 3 |  |  | 5 |  |  |  |  |  | LF |  |  |  |  | 3 |
| L#3 | 40 |  | 18 |  | 24 |  |  | 30 |  | 0 |  |  | 5 |  |  |  |  |  | RR |  |  |  |  | 1 |
| L#4 | 22 |  | 19 |  | 23 |  |  | 10 |  | 0 |  |  | 2 |  |  |  |  |  | LR |  |  |  |  | 3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wear Limits |  |  |  |  | ¼” each side ½” total |
| R#1 | 38 |  | 21 |  | 20 |  |  | 28 |  | 0 |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
| R#2 | 38 |  | 19 |  | 20 |  |  | 28 |  | 3 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| R#3 | 40 |  | 17 |  | 22 |  |  | 30 |  | 0 |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |
| R#4 | 21 |  | 19 |  | 22 |  |  | 10 |  | 0 |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
| Wear Limits | 17/16” MIN 16/16” Fed |  | 23/16” MAX 24/16” Fed |  | 15/16” MIN 14/16” Fed |  |  | 8/16” MIN |  | 8/16” |  |  | 8/16” |  |  |  |  |  |  |  |  |  |  |  |
|  | Coupler Slack (229.61(a)(2)(4)) |  | Coupler Height (232.2) |  |  | Pilot Height (229.123) |  | Coupler Swing |  |  |  |  |  | Condition of Springs (229.65) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Left |  |  |  |  |  | Right |  |  |  |  |
| Front |  |  |  |  |  |  |  |  |  |  |  |  |  | ## F |  | Good |  |  |  | ## F |  | Good |  |  |
| Rear |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wear Limits |  | 8/16 |  | 31 ½” 34 ½” |  |  | 3” 6” |  | 8” 18” |  | MIN MAX |  |  |  | ## R |  | Good |  |  |  | ## R |  | Good |  |

Comments:

All measurements in 16th of an inch.

All traction motor leads cut at tm clamps and carbody clamps.

All cabling and wiring stripped from above frame and inside electrical cabinet.

Some contactors missing from HV cabinet.

All Journal bearings are Hyatt 6 ½ x 12 boxes..

Wheel Sets (FRA Part 229.73)

- (229.73(a)) Variation between wheels on the same axle may not exceed 2 tape sizes when applied or turned.

- (229.73(b)) Variation between wheel diameter on the same SD truck may not exceed 12/16” without

Shims and 20/16” with shims.

- (229.73(b)) Variation between wheel diameters on different SD trucks may not exceed 20/16”.