# WHEEL & TRUCK REPORT				LOCOMOTIVE:  ACWR 9529

# EMPLOYEE:       Johny Radke

# ACWR  Form 7070						DATE: 	   June 13, 2022

|  | Wheel Thickness (229.75(j)) |  | Flange Height (229.75(h)) |  | Flange Thickness (229.75(f)) |  |  | Witness Reading (229.73) |  | Shims Installed (229.73(b)) |  |  | Pedestal Jaw Wear |  |  |  |  |  | Side Bearing Clearance (229.69(b)) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L#1 | 51 |  | 20 |  | 18 |  |  | 40 |  | 0 |  |  | 3 |  |  |  |  |  | RF |  |  |  |  | 5 |
| L#2 | 47 |  | 20 |  | 21 |  |  | 36 |  | 0 |  |  | 3 |  |  |  |  |  | LF |  |  |  |  | 1 |
| L#3 | 26 |  | 19 |  | 20 |  |  | 15 |  | 0 |  |  | 7 |  |  |  |  |  | RR |  |  |  |  | 5 |
| L#4 | 38 |  | 20 |  | 21 |  |  | 27 |  | 0 |  |  | 3 |  |  |  |  |  | LR |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wear Limits |  |  |  |  | ¼” each side ½” total |
| R#1 | 51 |  | 17 |  | 21 |  |  | 40 |  | 0 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| R#2 | 49 |  | 18 |  | 18 |  |  | 38 |  | 0 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| R#3 | 26 |  | 21 |  | 19 |  |  | 15 |  | 0 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| R#4 | 38 |  | 19 |  | 21 |  |  | 27 |  | 0 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| Wear Limits | 17/16” MIN 16/16” Fed |  | 23/16” MAX 24/16” Fed |  | 15/16” MIN 14/16” Fed |  |  | 8/16” MIN |  | 8/16” |  |  | 8/16” |  |  |  |  |  |  |  |  |  |  |  |
|  | Coupler Slack (229.61(a)(2)(4)) |  | Coupler Height (232.2) |  |  | Pilot Height (229.123) |  | Coupler Swing |  |  |  |  |  | Condition of Springs (229.65) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Left |  |  |  |  |  | Right |  |  |  |  |
| Front | 0 |  | 32 |  |  | 6 |  | 4 |  |  |  |  |  | ## F |  | Good |  |  |  | ## F |  | Good |  |  |
| Rear | 0 |  | 32 |  |  | 4 1/2 |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wear Limits |  | 8/16 |  | 31 ½” 34 ½” |  |  | 3” 6” |  | 8” 18” |  | MIN MAX |  |  |  | ## R |  | Good |  |  |  | ## R |  | Good |  |

Comments:

All measurements in 16th of an inch.

Right hand side bearing clearance excessive.

L4 wheel chipped / skids noted. (Serviceable)

All Journal bearings are Hyatt 6 ½ x 12 boxes.

Wheel Sets (FRA Part 229.73)

- (229.73(a)) Variation between wheels on the same axle may not exceed 2 tape sizes when applied or turned.

- (229.73(b)) Variation between wheel diameter on the same SD truck may not exceed 12/16” without

Shims and 20/16” with shims.

- (229.73(b)) Variation between wheel diameters on different SD trucks may not exceed 20/16”.